ARDUINO CODE

#include <ESP8266WiFi.h>

// ================= WIFI =================

const char* ssid = "JioFiber-Kedar4G";

const char* password = "kedarnath";

WiFiServer server(9999);

// ================= PIN CONFIG =================

#define IN1 5    // D1

#define IN2 4    // D2

#define IN3 12   // D6

#define IN4 13   // D7

#define ENA 14   // D5

#define ENB 15   // D8

char command;

// 🔥 Reduced speed (0–1023)

int speedValue = 100;

// ================= SETUP =================

void setup() {

Serial.begin(9600);

pinMode(IN1, OUTPUT);

pinMode(IN2, OUTPUT);

pinMode(ENA, OUTPUT);

pinMode(IN3, OUTPUT);

pinMode(IN4, OUTPUT);

pinMode(ENB, OUTPUT);

stopMotors();

Serial.println("Connecting to WiFi...");

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nConnected!");

Serial.print("IP Address: ");

Serial.println(WiFi.localIP());

server.begin();

}

// ================= MOTOR CONTROL =================

// LEFT MOTOR

void leftForward(int speed) {

digitalWrite(IN1, HIGH);

digitalWrite(IN2, LOW);

analogWrite(ENA, speed);

}

void leftBackward(int speed) {

digitalWrite(IN1, LOW);

digitalWrite(IN2, HIGH);

analogWrite(ENA, speed);

}

// RIGHT MOTOR (REVERSED FIX)

void rightForward(int speed) {

digitalWrite(IN3, LOW);   // reversed

digitalWrite(IN4, HIGH);

analogWrite(ENB, speed);

}

void rightBackward(int speed) {

digitalWrite(IN3, HIGH);  // reversed

digitalWrite(IN4, LOW);

analogWrite(ENB, speed);

}

// ================= MOVEMENTS =================

void moveForward() {

Serial.println("Forward");

leftForward(speedValue);

rightForward(speedValue);

}

void moveBackward() {

Serial.println("Backward");

leftBackward(speedValue);

rightBackward(speedValue);

}

void turnLeft() {

Serial.println("Left");

leftBackward(speedValue);

rightForward(speedValue);

}

void turnRight() {

Serial.println("Right");

leftForward(speedValue);

rightBackward(speedValue);

}

void stopMotors() {

Serial.println("Stop");

analogWrite(ENA, 0);

analogWrite(ENB, 0);

digitalWrite(IN1, LOW);

digitalWrite(IN2, LOW);

digitalWrite(IN3, LOW);

digitalWrite(IN4, LOW);

}

// ================= LOOP =================

void loop() {

WiFiClient client = server.available();

if (client) {

Serial.println("Client Connected");

while (client.connected()) {

if (client.available()) {

command = client.read();

if (command == '\n' || command == '\r') continue;

Serial.print("Received: ");

Serial.println(command);

switch (command) {

case 'F': moveForward(); break;

case 'B': moveBackward(); break;

case 'L': turnLeft(); break;

case 'R': turnRight(); break;

case 'S': stopMotors(); break;

}

}

}

client.stop();

Serial.println("Client Disconnected");

}

}

vsCODE

import cv2

from ultralytics import YOLO

import socket

import time

# ===================== ESP8266 CONNECTION =====================

ESP_IP = "192.168.29.128"   # 🔴 your ESP IP (NO SPACE)

PORT = 9999

def connect_esp():

while True:

try:

client = socket.socket(socket.AF_INET, socket.SOCK_STREAM)

client.connect((ESP_IP, PORT))

print("Connected to ESP8266")

return client

except:

print("Retrying ESP connection...")

time.sleep(2)

client = connect_esp()

# ===================== LOAD MODEL =====================

model = YOLO("yolov8n.pt")

# ===================== CAMERA =====================

cap = cv2.VideoCapture("http://192.168.29.39:8080/video", cv2.CAP_FFMPEG)

cap.set(cv2.CAP_PROP_BUFFERSIZE, 1)

# ===================== SETTINGS =====================

last_action = ""

CONF_THRESHOLD = 0.5

frame_count = 0

# ===================== MAIN LOOP =====================

while True:

ret, frame = cap.read()

if not ret:

print("Camera not working")

break

# 🔥 Resize for speed

frame = cv2.resize(frame, (320, 240))

frame_count += 1

# 🔥 Skip frames

if frame_count % 3 != 0:

cv2.imshow("AutoCar AI", frame)

if cv2.waitKey(1) == 27:

break

continue

h, w, _ = frame.shape

center_frame = w // 2

action = "F"  # default forward

total_area = 0

# ===================== YOLO DETECTION =====================

results = model(frame, imgsz=320, conf=CONF_THRESHOLD, verbose=False)

for r in results:

for box in r.boxes:

cls = int(box.cls[0])

label = model.names[cls]

x1, y1, x2, y2 = map(int, box.xyxy[0])

center_x = (x1 + x2) // 2

box_area = (x2 - x1) * (y2 - y1)

total_area += box_area

# Draw box

cv2.rectangle(frame, (x1, y1), (x2, y2), (0,255,0), 2)

cv2.putText(frame, label, (x1, y1-10),

cv2.FONT_HERSHEY_SIMPLEX, 0.5, (0,255,0), 2)

# ===================== DECISION =====================

# 🚨 VERY CLOSE OBJECT

if box_area > 50000:

action = "S"

break

# LEFT → TURN RIGHT

elif center_x < center_frame - 80:

action = "R"

# RIGHT → TURN LEFT

elif center_x > center_frame + 80:

action = "L"

# ===================== WALL DETECTION =====================

gray = cv2.cvtColor(frame, cv2.COLOR_BGR2GRAY)

edges = cv2.Canny(gray, 50, 150)

edge_count = cv2.countNonZero(edges)

# 🚨 TOO MANY EDGES → WALL CLOSE

if edge_count > 15000:

action = "S"

# 🚨 TOO MUCH AREA → SOMETHING CLOSE

if total_area > 90000:

action = "S"

# ===================== SEND COMMAND =====================

if action != last_action:

try:

client.send((action + "\n").encode())

last_action = action

print("Action:", action)

except Exception as e:

print("WiFi Error:", e)

client = connect_esp()

# ===================== DISPLAY =====================

cv2.putText(frame, f"ACTION: {action}",

(10, 30),

cv2.FONT_HERSHEY_SIMPLEX,

0.8,

(0,0,255),

2)

cv2.imshow("AutoCar AI", frame)

# ESC to exit

if cv2.waitKey(1) == 27:

break

# ===================== CLEANUP =====================

cap.release()

cv2.destroyAllWindows()

client.close()